FLHubTool 
User Guide

Revision History

| Revision | Date | Comment |
| --- | --- | --- |
| 0.1 | 24 April 2015 | Initial release |
| 1.0 | 21 August 2015 | Add command line mode. |
| 1.1 | 16 November 2015 | GUI update. |
| 1.2 | 3 August 2015 | Installation update. |
| 1.3 | 30, January, 2017 | Update header/footer |
| 1.4 | 27, November, 2017 | Add command line description. |

Table of Contents

1	General Overview	4

2	System Environment	5

3	Package Components	6

4	Command Line	7

4.1	Display Help	9

4.2	List Fresco Logic hub and devices.	9

4.3	FL5000 Hub EEPROM Version Display.	9

4.4	FL5000 Hub EEPROM Dump.	10

4.5	FL5000 Hub EEPROM Update.	10

4.6	FL5000 Hub EEPROM Erase.	10

4.7	FL5000 Hub EEPROM CheckSum.	10

4.8	FL5000 Hub Firmware Version Display.	11

4.9	FL5000 Hub Firmware Dump.	11

4.10	FL5000 Hub Firmware Update.	11

4.11	FL5000 Hub Firmware Erase.	11

4.12	FL5000 + 7100 Device Firmware Version Display.	11

4.13	FL5000 + 7100 Device Firmware Dump.	12

4.14	FL5000 + 7100 Device Firmware Update.	12

4.15	FL5000 + 7100 Device Firmware Erase.	12

4.16	FL7100 Device Firmware Version Display.	12

4.17	FL7100 Device Firmware Dump.	12

4.18	FL7100 Device Firmware Update.	12

4.19	FL7100 Device Firmware Erase.	13

General Overview

FLHubTool is a multiple function windows application for Fresco Logic USB 3.1 Gen1 Host, USB 3.1 Gen1/2 Hub and USB 3.1 PD Controller product EEPROM and firmware operation.

System Environment

Software requirements:

Operating Systems – Microsoft® Windows® 8 or later

Hardware requirements:

Machine with USB 3.0 xHCI Host Controller. (Please use Microsoft USB 3.0 inbox driver to prevent programming issue.)

Package Components

The following table lists the FLHubTool components.

| Component | Explanation |
| --- | --- |
| FLHubTool.exe | The FLHubTool Windows Application. |

Command Line

Please open a command line prompt and type the following command:

FLHubTool –HELP

Then it will find the following usage:

===FLHubTool v16.6e===

(c)Copyright 2017, Fresco Logic, Incorporated.

Usage :

01. Display Help.

Command : -HELP

02. List Fresco Logic hub and devices.

Command : -LIST

03. FL5000 Hub EEPROM Version Display.

Ex. For Hub 0, Command : -DEV_0 -EEPROM -VERSION

Ex. For Hub n, Command : -DEV_n -EEPROM -VERSION

Ex. For Hub n with 24C04EEPROM, Command : -DEV_n -EEPROM -VERSION -24C04

04. FL5000 Hub EEPROM Dump.

Ex. For Hub 0, Command : -DEV_0 -EEPROM -DUMP

Ex. For Hub 0 with 24C04EEPROM, Command : -DEV_0 -EEPROM -DUMP -24C04

05. FL5000 Hub EEPROM Update.

Ex. For Hub 0, Command : -DEV_0 -EEPROM -UPDATE FileName.bin

Ex. For Hub 0 with 24C04EEPROM, Command : -DEV_0 -EEPROM -24C04 -UPDATE FileName.bin

06. FL5000 Hub EEPROM Erase.

Ex. For Hub 0, Command : -DEV_0 -EEPROM -ERASE

Ex. For Hub 0 with 24C04EEPROM, Command : -DEV_0 -EEPROM -ERASE -24C04

07. FL5000 Hub EEPROM CheckSum.

Ex. For Hub 0, Command : -DEV_0 -EEPROM -CHECKSUM

Ex. For Hub 0 with 24C04EEPROM, Command : -DEV_0 -EEPROM -CHECKSUM -24C04

08. FL5000 Hub Firmware Version Display.

Ex. For Hub 0, Command : -DEV_0 -FIRMWARE -VERSION

Ex. For Hub n, Command : -DEV_n -FIRMWARE -VERSION

09. FL5000 Hub Firmware Dump.

Ex. For Hub 0, Command : -DEV_0 -FIRMWARE -DUMP

10. FL5000 Hub Firmware Update.

Ex. For Hub 0, Command : -DEV_0 -FIRMWARE -UPDATE FileName.bin

11. FL5000 Hub Firmware Erase.

Ex. For Hub 0, Command : -DEV_0 -FIRMWARE -ERASE

12. FL5000 + 7100 Device Firmware Version Display.

Ex. For Device 0, Command : -DEV_0 -FIRMWARE_5KPC -VERSION

Ex. For Device n, Command : -DEV_n -FIRMWARE_5KPC -VERSION

13. FL5000 + 7100 Device Firmware Dump.

Ex. For Device 0, Command : -DEV_0 -FIRMWARE_5KPC -DUMP

14. FL5000 + 7100 Device Firmware Update.

Ex. For Device 0, Command : -DEV_0 -FIRMWARE_5KPC -UPDATE FileName.bin

15. FL5000 + 7100 Device Firmware Erase.

Ex. For Device 0, Command : -DEV_0 -FIRMWARE_5KPC -ERASE

16. FL7100 Device Firmware Version Display.

Ex. For Device 0, Command : -DEV_0 -FIRMWARE_PC -VERSION

Ex. For Device n, Command : -DEV_n -FIRMWARE_PC -VERSION

17. FL7100 Device Firmware Dump.

Ex. For Device 0, Command : -DEV_0 -FIRMWARE_PC -DUMP

18. FL7100 Device Firmware Update.

Ex. For Device 0, Command : -DEV_0 -FIRMWARE_PC -UPDATE FileName.bin

19. FL7100 Device Firmware Erase.

Ex. For Device 0, Command : -DEV_0 -FIRMWARE_PC -ERASE

Please wait, uninitializing...Done.

Display Help

Command: FLHubTool -HELP

Display the usage of command line mode.

List Fresco Logic hub and devices.

Command: FLHubTool -LIST

Display known Fresco Logic FL5000 Hub and FL7100 Device.

FL5000 Hub EEPROM Version Display.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –EERPOM –VERSION

For EEPROM model 24C04.

Command: FLHubTool –DEV_0 –EEPROM –VERSION –24C04

FL5000 Hub EEPROM Dump.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –EEPROM –DUMP

For EEPROM model 24C04.

Command: FLHubTool –DEV_0 –EEPROM –DUMP -24C04

FL5000 Hub EEPROM Update.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –EEPROM –UPDATE FileName.bin

For EEPROM model 24C04

Command: FLHubTool –DEV_0 –EEPROM –UPDATE FileName.bin -24C04

FL5000 Hub EEPROM Erase.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –EEPROM –ERASE

For EEPROM model 24C04

Command: FLHubTool –DEV_0 –EEPROM –ERASE -24C04

FL5000 Hub EEPROM CheckSum.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –EEPROM –CHECK_SUM

For EEPROM model 24C04

Command: FLHubTool –DEV_0 –EEPROM –CHECK_SUM -24C04

FL5000 Hub Firmware Version Display.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –FIRMWARE -VERSION

FL5000 Hub Firmware Dump.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –FIRMWARE -DUMP

FL5000 Hub Firmware Update.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –FIRMWARE –UPDATE FileName.bin

FL5000 Hub Firmware Erase.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –FIRMWARE –ERASE

FL5000 + 7100 Device Firmware Version Display.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –FIRMWARE_5KPC –VERSION

FL5000 + 7100 Device Firmware Dump.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –FIRMWARE_5KPC –VERSION

FL5000 + 7100 Device Firmware Update.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –FIRMWARE_5KPC –UPDATE FileName.bin

FL5000 + 7100 Device Firmware Erase.

DEV_0 is the device node of FL5000.

Command: FLHubTool –DEV_0 –FIRMWARE_5KPC –ERASE

FL7100 Device Firmware Version Display.

DEV_0 is the device node of FL7100.

Command: FLHubTool –DEV_0 –FIRMWARE_PC -VERSION

FL7100 Device Firmware Dump.

DEV_0 is the device node of FL7100.

Command: FLHubTool –DEV_0 –FIRMWARE_PC –DUMP

FL7100 Device Firmware Update.

DEV_0 is the device node of FL7100.

Command: FLHubTool –DEV_0 –FIRMWARE_PC –UPDATE FileName.bin

FL7100 Device Firmware Erase.

DEV_0 is the device node of FL7100.

Command: FLHubTool –DEV_0 –FIRMWARE_PC –ERASE

Legal Disclaimer

Information in this document is provided in connection with Fresco Logic, Inc. products. No license, express or implied, by estoppels or otherwise, to any intellectual property rights is granted by this document. Except as provided in Fresco Logic’s terms and conditions of sale for such products, Fresco Logic assumes no liability whatsoever, and Fresco Logic disclaims any express or implied warranty, relating to sale and/or use of Fresco Logic products, including liability or warranties relating to fitness for a particular purpose, merchantability, or infringement of any patent, copyright or other intellectual property right. This document provides technical information for the user. Fresco Logic, Inc. reserves the right to modify the information in this document as necessary. The customer should make sure that they have the most recent datasheet version. Fresco Logic, Inc. holds no responsibility for any errors that may appear in this document. Customers should take appropriate action to ensure their use of the products does not infringe upon any patents. Fresco Logic, Inc. respects valid patent right of third parties and does not infringe upon or assist others to infringe upon such rights. All information contained herein is subject to change without notice.